Project Sunflower

ESP32 Based Rhythm Game

Overview: This project implements a reaction-based rhythm game using a microcontroller. Audio playback is synchronized with LED matrix animations. The system processes user inputs via GPIO interrupts and evaluates the timing accuracy to generate scores based on performance.

Purpose: To design an interactive system that integrates real-time input processing, audio output, and visual feedback to create an engaging and enjoyable experience for the user

Requirements:

- Handle button inputs using interrupts for real-time response

- Display incoming notes using an LED matrix

- Provide music playback and level selection

- Generate sound output through a speaker module

- Calculate and display player score

- Support multiple difficulty levels

- Able to store and read MP3 or Wav files

- Ensure system is portable and self-contained

Architecture: The system is divided into the following modules:

- Game Logic Module:

  - A central timing engine

  - Controls timing, note generation, visual and audio synchronization, scoring system, and difficulty levels

  - Microcontroller handles game logic

- Input handler:

  - Processes button presses and joystick input.

  - Uses GPIO interrupt handling

- LED Controller:

  - Drives the 64x32 RGB LED matrix to display scrolling notes

  - Uses SPI high speed data transfer

- Audio Output Module:

  - Generates music signals and outputs through the speaker

  - Uses PWM-based audio output

- Display Module:

  - Displays menus, score display, and game status

  - Uses I2C

- Feedback System:

  - Provides visual or audio feedback based on player performance

Diagram:

Hardware design:

- Main controller: ESP32C6 Microcontroller

- Main game display: SMD2121 64x32 Pixel RGB LED Matrix

- Audio output: 8Ω 3W Mini Speaker

- Player input: Tactile Push Buttons

- UI display: SSD1306 OLED Display

- UI navigation: Analog Joystick

- Storage: Memory Card Adapter & Micro SD Card

Key Challenges:

- Timing synchronization between audio and LED animation

- Driving the large LED matrix on limited resources

- Managing tasks using a real-time scheduling approach

- Limited pins on the ESP32C6

Future Enhancements:

- Battery-powered system with 2–3 hours runtime

- Wireless updates via WiFi (OTA or server sync for new songs)

- Optional Bluetooth audio output for improved sound quality